# Project Description: Simulated Distribution Station Control

# Overview

This project involves developing and implementing an industrial automation control system for a simulated Distribution Station. The primary function of the station is to store raw workpieces and transfer them reliably to downstream production processes within a modular, distributed manufacturing environment.

# System Architecture & Subsystems

The station is divided into two primary, interconnected subsystems that must execute coordinated control sequences:

## 1. Stack Magazine Subsystem

- Function: Stores workpieces in a vertical column and dispenses them one by one.

- Controller Responsibilities:

  - Empty sensor

  - Cylinder limit position sensors

  - Commands the cylinder to feed a workpiece

## 2. Transfer Unit Subsystem

- Function: Picks up the separated workpiece at the handover point and transports it to the defined output location for the next station.

- Controller Responsibilities:

  - Coordinates a vacuum gripper to grip and release parts safely.

  - Controls transfer drive actuators to transition between pickup and drop-off positions.

  - Motion limit switches

  - Vacuum sensor

## 3. Requirements

### 3.1 General Requirements

GR-01 Station should work in MANUAL and AUTOMATIC state.

### 3.2 Initial Requirements

IR-01 At Initial state Stack Magazine cylinder is retracted

IR-02 At Initial state Transfer Unit Arm is at magazine point and vacuum is turned off

IR-03 After system is turned on , station goes to initial position

### 3.3 Automatic Mode Requirements

Those requirements are suitable for automatic mode:

AMR-01 When Stack Magazine and Transfer Unit are in Initial position , Ready indicator should activate(Start button led turn on)

AMR-02 When Start button is pressed , station should start(continue) working cycle

AMR-03 When Stop is triggered, station should continue to next stable state and then stop working cycle

AMR-04 When station is stopped and RESET is triggered, station and acknowledge indication should go to initial state

AMR-05 Transition from Manual to Automatic is only possible when station is stopped

AMR-05 Transition from Automatic to Manual is only possible when station complete all commands

AMR-06 If Magazine stack is not empty, Acknowledge button led should turn on

AMR-07 If Magazine stack is empty, Acknowledge button should start blink

AMR-08 Acknowledge button stops blinking, after operator pressed acknowledge button

### 3.4 Stack Magazine Requirements

MR-01 if sensor is empty for longer then 1.5 second, stack empty indicator should activate(ack blinking)

### 3.5 Transfer Unit Requirements

TR-01 Vacuum should not turn off during transportation of workpiece between stable points

### 3.6 Manual Mode Requirements

During manual operation next functionalities are available:

MMR-01: When Push WP is active, Stack Magazine cylinder is extending

MMR-02: When Push WP is not active, Stack Magazine cylinder is retracting

MMR-03 When GO TO MAG is active, arm goes to magazine point

MMR-04 When GO TO NEXT is active, arm goes to next station point

MMR-05 When both GO TO MAG and GO TO NEXT is active, arm stops

MMR-06 When Vacuum on is active, vacuum is active

MMR-07 When Vacuum off is active, vacuum is not active

MMR-08 When both VACUUM ON and VACUUM OFF  is active, vacuum is not active

## 4.Logical Design